## What is groundwater?

It is sometimes thought that water flows through underground rivers or that it collects in underground lakes. Groundwater is not confined to only a few channels or depressions in the same way that surface water is concentrated in streams and lakes. Rather, it exists almost everywhere underground. It is found underground in the spaces between particles of rock and soil, or in crevices and cracks in rock. The water filling these openings is usually within 100 metres of the surface. Much of the earth's fresh water is found in these spaces. At greater depths, because of the weight of overlying rock, these openings are much smaller, and therefore hold considerably smaller quantities of water.

Groundwater flows slowly through water-bearing formations (aquifers) at different rates. In some places, where groundwater has dissolved limestone to form caverns and large openings, its rate of flow can be relatively fast but this is exceptional.

Many terms are used to describe the nature and extent of the groundwater resource. The level below which all the spaces are filled with water is called the water table. Above the water table lies the unsaturated zone. Here the spaces in the rock and soil contain both air and water. Water in this zone is called soil moisture. The entire region below the water table is called the saturated zone, and water in this saturated zone is called groundwater.

## What is an aquifer?

Although groundwater exists everywhere under the ground, some parts of the saturated zone contain more water than others. An aquifer is an underground formation of permeable rock or loose material which can produce useful quantities of water when tapped by a well. Aquifers come in all sizes. They may be small, only a few hectares in area, or very large, underlying thousands of square kilometres of the earth's surface. They may be only a few metres thick, or they may measure hundreds of metres from top to bottom.

## Groundwater – Always on the move

Permeable material contains interconnected cracks or spaces that are both numerous enough and large enough to allow water to move freely. In some permeable materials groundwater may move several metres in a day; in other places, it moves only a few centimetres in a century. Groundwater moves very slowly through relatively impermeable materials such as clay and shale.

Groundwater scientists generally distinguish between two types of aquifers in terms of the physical attributes of the aquifer: porous media and fractured aquifers.

Porous media are those aquifers consisting of aggregates of individual particles such as sand or gravel. The groundwater occurs in and moves through the openings between the individual grains. Porous media where the grains are not connected to each other are considered unconsolidated. If the grains are cemented together, such aquifers are called consolidated. Sandstones are examples of consolidated porous media.

Fractured aquifers are rocks in which the groundwater moves through cracks, joints or fractures in otherwise solid rock. Examples of fractured aquifers include granite and basalt. Limestones are often fractured aquifers, but here the cracks and fractures may be enlarged by solution, forming large channels or even caverns. Limestone terrain where solution has been very active is termed karst. Porous media such as sandstone may become so highly cemented or recrystallized that all of the original space is filled. In this case, the rock is no longer a porous medium. However, if it contains cracks it can still act as a fractured aquifer.

Most of the aquifers of importance to us are unconsolidated porous media such as sand and gravel. Some very porous materials are not permeable. Clay, for instance, has many spaces between its grains, but the spaces are not large enough to permit free movement of water.

Groundwater usually flows downhill with the slope of the water table. Like surface water, groundwater flows toward, and eventually drains into streams, rivers, lakes and the oceans. Groundwater flow in the aquifers underlying surface drainage basins, however, does not always mirror the flow of water on the surface. Therefore, groundwater may move in different directions below the ground than the water flowing on the surface.

Unconfined aquifers are those that are bounded by the water table. Some aquifers, however, lie beneath layers of impermeable materials. These are called confined aquifers, or sometimes artesian aquifers. A well in such an aquifer is called an artesian well. The water in these wells rises higher than the top of the aquifer because of confining pressure. If the water level rises above the ground surface a flowing artesian well occurs. The piezometric surface is the level to which the water in an artesian aquifer will rise.

## Groundwater – A major link in the hydrologic cycle

Groundwater circulates as part of the hydrologic cycle. As precipitation and other surface water sources recharge the groundwater it drains steadily, and sometimes very slowly, towards its discharge point.

Groundwater does not stay underground forever, and it does not lie still waiting for us to draw it from a well. The hydrologic cycle is the series of transformations that occur in the circulation of water from the atmosphere onto the surface and into the subsurface regions of the earth, and then back from the surface to the atmosphere. Precipitation becomes surface water, soil moisture, and groundwater. Groundwater circulates back to the surface, and from the surface all water returns to the atmosphere through evaporation and transpiration.

When precipitation falls on the land surface, part of the water runs off into the lakes and rivers. Some of the water from melting snow and from rainfall seeps into the soil and percolates into the saturated zone. This process is called recharge (Figure 2). Places where recharge occurs are referred to as recharge areas.

Eventually, this water reappears above the ground. This is called discharge. Groundwater may flow into streams, rivers, marshes, lakes and oceans, or it may discharge in the form of springs and flowing wells.

Groundwater discharge can contribute significantly to surface water flow. In dry periods, the flow of some streams may be supplied entirely by groundwater. At all times of the year, in fact, the nature of underground formations has a profound effect on the volume of surface runoff. While the rate of discharge determines the volume of water moving from the saturated zone into streams, the rate of recharge determines the volume of water running over the surface. When it rains, for instance, the volume of water running into streams and rivers depends on how much rainfall the underground materials can absorb. When there is more water on the surface than can be absorbed into the groundwater zone, it runs off into streams and lakes.

The residence time of groundwater, i.e., the length of time water spends in the groundwater portion of the hydrologic cycle, varies enormously. Water may spend as little as days or weeks underground, or as much as 10 000 or more years. Residence times of tens, hundreds, or even thousands of years are not unusual. By comparison, the average turnover time of river water, or the time it takes the water in rivers to completely replace itself, is about two weeks

According to some estimates, the quantity of groundwater in the earth would cover the entire surface of the globe to a depth of 120 metres. By contrast, the volume of surface water in lakes, rivers, reservoirs and swamps could be contained in a depth of about one quarter of a metre.

It is extremely difficult to estimate the volume of groundwater on the entire planet. For example, a recent review of the literature revealed estimated figures ranging from 7 000 000 to 330 000 000 cubic kilometres. However, all the estimates imply that if we do not include the water frozen in ice caps, glaciers and permanent snow, groundwater makes up almost the entire volume of the earth's usable fresh water. Yet, this supply is often not easily accessible, and it may be difficult and expensive to develop these water supplies in some regions. The quality of the groundwater source is also a significant determining factor when identifying its use.

## Groundwater quality

We often think of water quality as a matter of taste, clarity and odour, and in terms of other properties which determine whether water is fit for drinking. For other uses different properties may be important. Most of these properties depend on the kinds of substances that are dissolved or suspended in the water. Water for most industrial uses, for instance, must not be corrosive and must not contain dissolved solids that might precipitate on the surfaces of machinery and equipment.

Pure water is tasteless and odourless. A molecule of water contains only hydrogen and oxygen atoms. Water is never found in a pure state in nature. Both groundwater and surface water may contain many constituents, including microorganisms, gases, inorganic and organic materials.

The chemical nature of water continually evolves as it moves through the hydrologic cycle. The kinds of chemical constituents found in groundwater depend, in part, on the chemistry of the precipitation and recharge water. Near coastlines, precipitation contains higher concentrations of sodium chloride, and downwind of industrial areas, airborne sulphur and nitrogen compounds make precipitation acidic.

One of the most important natural changes in groundwater chemistry occurs in the soil. Soils contain high concentrations of carbon dioxide which dissolves in the groundwater, creating a weak acid capable of dissolving many silicate minerals. In its passage from recharge to discharge area, groundwater may dissolve substances it encounters or it may deposit some of its constituents along the way. The eventual quality of the groundwater depends on temperature and pressure conditions, on the kinds of rock and soil formations through which the groundwater flows, and possibly on the residence time. In general, faster flowing water dissolves less material. Groundwater, of course, carries with it any soluble contaminants which it encounters.

Scientists assess water quality by measuring the amounts of the various constituents contained in the water. These amounts are often expressed as milligrams per litre (mg/L), which is equivalent to the number of grams of a substance per million grams of water.

The suitability of water for a given use depends on many factors such as hardness, salinity and pH. Acceptable values for each of these parameters for any given use depend on the use, not on the source of the water, so that the considerations important for surface water (as mentioned in Freshwater Series No. A-3, entitled "Clean Water – Life Depends on It!") are equally applicable to groundwater.

The natural quality of groundwater differs from surface water in that:

- for any given source, its quality, temperature and other parameters are less variable over the course of time; and,

- in nature, the range of groundwater parameters encountered is much larger than for surface water, e.g., total dissolved solids can range from 25 mg/L in some places in the Canadian Shield to 300 000 mg/L in some deep saline waters in the Interior Plains.

At any given location, groundwater tends to be harder and more saline than surface water, but this is by no means a universal rule. It is also generally the case that groundwater becomes more saline with increasing depth, but again, there are many exceptions.

As groundwater flows through an aquifer it is naturally filtered. This filtering, combined with the long residence time underground, means that groundwater is usually free from disease-causing microorganisms. A source of contamination close to a well, however, can defeat these natural safeguards. Natural filtering also means that groundwater usually contains less suspended material and undissolved solids than surface water.

Aquifer

in hydrology, rock layer that contains water and releases it in appreciable amounts. The rock contains water-filled pore spaces, and, when the spaces are connected, the water is able to flow through the matrix of the rock. An aquifer also may be called a water-bearing stratum, lens, or zone.

A confined aquifer is a water-bearing stratum that is confined or overlain by a rock layer that does not transmit water in any appreciable amount or that is impermeable. There probably are few truly confined aquifers, because tests have shown that the confining strata, or layers, although they do not readily transmit water, over a period of time contribute large quantities of water by slow leakage to supplement production from the principal aquifer.

A groundwater aquifer is said to be unconfined when its upper surface (water table) is open to the atmosphere through permeable material. As opposed to a confined aquifer, the water table in an unconfined aquifer system has no overlying impervious rock layer to separate it from the atmosphere.

Spring in hydrology is an opening at or near the surface of the Earth for the discharge of water from underground sources. A spring is a natural discharge point of subterranean water at the surface of the ground or directly into the bed of a stream, lake, or sea. Water that emerges at the surface without a perceptible current is called a seep. Wells are holes excavated to bring water and other underground fluids to the surface.

Water in springs, seeps, and wells generally originates as rainfall that has soaked into the soil and percolated into underlying rocks. Permeable rocks (those containing interconnected pore spaces through which water can migrate), such as limestone and sandstone, store and transmit water and are called aquifers. Sometimes the water in an aquifer becomes confined between two impermeable rock layers, such as clay or shale. When these strata are tilted or folded into a structural trap, the water in the lower part of the aquifer is stored under pressure. If the pressure is sufficiently high and a well is sunk through the capping layer, the water will rise to the surface without pumping. This is called an artesian well.

The aquifers that receive and discharge the largest quantities of groundwater are unconsolidated materials such as sand and gravel. These aquifers occur widely and discharge groundwater partly through springs, but mostly through evaporation and seepage. Well water is obtained mostly from such aquifers, particularly those underlying lowland areas. In limestone regions, rainwater sinks through holes or other openings and is drained chiefly through underground passages. Basalt and sandstone aquifers are also sources of water for many springs. Most of the springs whose individual discharges exceed 3 cubic metres per second (100 cubic feet per second) originate from limestone and basalt aquifers.

Springs can be classified by their water temperature. A thermal or hot spring has a water temperature significantly higher than the mean air temperature of the surrounding area. Thermal springs occur in volcanic regions and in areas where rock layers have been broken and folded in geologically recent time. Geysers, a spectacular form of hot spring, eject tall plumes of hot water and steam. Springs containing a noticeable amount of dissolved substances are called mineral springs. Most thermal springs are rich in dissolved minerals while many mineral springs are warm.

The quality of water discharged by a spring depends on the type of aquifer and rock strata through which the water has passed, the temperatures along the route, and the volume of circulating water, past and present. Groundwater is least modified where it travels shallowly over short distances through pervious formations depleted of soluble minerals. Sand and gravel aquifers in humid regions yield good-quality water, whereaswater from springs and seeps in arid areas may be contaminated by undesirable mineral deposits. The water quality in sedimentary rocks of marine origin depends on the degree of freshwater flushing. After the brine has been washed away, limestone and sandstone rocks usually yield good-quality, though hard, freshwater.

fertile tract of land that occurs in a desert wherever a perennial supply of fresh water is available. Oases vary in size, ranging from about 1 hectare (2.5 acres) around small springs to vast areas of naturally watered or irrigated land. Underground water sources account for most oases; their springs and wells, some of them artesian, are supplied from sandstone aquifers whose intake areas may be more than 800 km (500 miles) away, as at al-Kh(rijah Oasis (Kharga) and ad-D(khilah Oasis (Dakhla) in the Libyan Desert. Two-thirds of the total population of the Sahara are sedentary peoples living in oases and depending on irrigation; these areas have temperatures conducive to rapid vegetative growth. In all Saharan oases the date palm constitutes the main source of food, while in its shade are grown citrus fruits, figs, peaches, apricots, vegetables, and cereals such as wheat, barley, and millet.

بَيْنَهُمَا